Allowing others to connect to my Consolidated wireless network

Step 1

Go to the computer you want to have access

Click Start

Click Run

Type what is between these brackets  [    cmd    ]  and Click OK

Type this [   ipconfig   /all    ]  and press Enter

Look below at the example

Write down the Physical Address (You may have more than one, but the only one that matters is under the Wireless section)

========================================================

Wireless Ethernet adapter Local Area Connection:

Connection-specific DNS Suffix  . : aol.com

Description . . . . . . . . . . . : Linksys LNE100TX Wireless Ethernet Adapter

Physical Address. . . . . . . . . : 00-12-17-5B-BE-DD

Dhcp Enabled. . . . . . . . . . . : Yes

Autoconfiguration Enabled . . . . : Yes

IP Address. . . . . . . . . . . . : 192.168.0.101

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . : 192.168.0.1

DHCP Server . . . . . . . . . . . : 192.168.0.1

DNS Servers . . . . . . . . . . . : 192.168.0.1

Lease Obtained. . . . . . . . . . : Monday, December 19, 2005 10:49:40 AM

Lease Expires . . . . . . . . . . : Monday, December 26, 2005 10:49:40 AM

========================================================

Your PDA, PSP, WiFi Phone, Printer, or other device may have the MAC Address on the device behind the battery or on the back of the device or card.

Step 2

Go to a computer that is already connected and working on the Internet

Click on Start

Click on Run

Type in [      iexplore   192.168.1.1      ] and click OK

Type in [   admin  ] as the User name

Type in [   cciadmin   ] as the password

Your password may be different

Click Wireless

Click Wireless MAC Filter (next line down)

Click Add

Put the Physical Address you wrote down earlier in an empty MAC Address box

*** USE COLONS NOT DASHES ***

Scroll to the bottom and choose Save Settings

It should be on the list now

Click the “X”

Close everything, reboot the computer that needed wireless and try to get in.